FORM D
CLINICAL ACCREDITATION GUIDELINES & CONDITIONALITIES
FOR PROSTHETICS AND ORTHOTICS PRACTICE IN NIGERIA

1. PURPOSE

This Form establishes the minimum clinical, technical, staffing and quality standards required for any facility providing Prosthetic and Orthotic (P&O) services in Nigeria, for the purposes of patient safety, professional training, public protection, and national/international recognition.

2. SCOPE

These standards apply to all Government hospitals, teaching hospitals, Specialist hospitals, Private clinics, Rehabilitation centres, and Non-governmental prosthetic/orthotic service providers operating within Nigeria.

3. PRINCIPLE OF ACCREDITATION

Accreditation shall be based on clinical capability/number of hospital beds, and the quality of service, rather than on the size of the institution or its ownership (public/private). Facilities shall be accredited only for the level of care they can safely provide.

4. ACCREDITATION LEVELS

All Prosthetic & Orthotic facilities shall be accredited under one of the following categories:

| Level | Classification |
| --- | --- |
| Level A | Primary Health Centre- Basic screening & simple aids like splints, shoe inserts |
| Level B | Secondary Health Centre- (Clinical P&O Centre); Specialist fitting & rehab |
| Level C (less than300 bed spaces) | Tertiary Health Centre (Advanced and complex P&O Centre), like myoelectric arms and advanced rehab |
| Level D (301 and above bed spaces) | National Training, Research & Referral Centre, like experimental and clinical trials, high-tech care, smart prostheses, and orthoses. |

5. AUTHORIZED SCOPE OF PRACTICE BY LEVEL

| Service / Capability | Level A | Level B | Level C | Level D |
| --- | --- | --- | --- | --- |
| Patient Assessment | Yes | Yes | Yes | Yes |
| Orthotic Fitting | Yes | Yes | Yes | Yes |
| Prosthetic Fitting | Yes | Yes | Yes | Yes |
| Maintenance and repairs | Yes | Yes | Yes | Yes |
| Socket Fabrication | No | Basic | Advanced | Advanced |
| Dynamic Alignment | Basic | Yes | Yes | Yes |
| Gait Training | Yes | Yes | Yes | Yes |
| Complex Cases (ulcers, paediatrics, bilateral, high-risk) | No | Limited | Yes | Yes |
| NYSC Staff | No | Yes | Yes | Yes |
| Internship Training | No | Yes | Yes | Yes |
| Degree Student Training | No | Yes | Yes | Yes |
| Research & Device Development | No | Limited | Yes | Yes |
| National Referral Services | No | No | Yes | Yes |

Facilities shall not operate outside their accredited scope of practice.

6. MINIMUM STAFFING REQUIREMENTS

Minimum staffing requirements by accreditation level are as follows:

| Level | Licensed Prosthetist/Orthotist (minimum) | Technicians (minimum) | Interns (minimum) | NYSC (minimum) | Administrative & support staff (minimum) |
| --- | --- | --- | --- | --- | --- |
| Level A | 1 | 1 | Nil | Optional | 1 |
| Level B | 2 | 2 | 1 | 1 | 2 |
| Level C (less than 300 bed spaces) | 3 | 3 | 3 | 2 | 3 |
| Level D (301 and above bed spaces) | 5 | 5 | 5 | 5 | 4 |

Leadership Requirement:

• Level A & B: Headed by a Clinical Prosthetist/Orthotist, with University first degree, and a minimum of five (5) years post-qualification experience.

• Level C & D: Headed by a Clinical Prosthetist/Orthotist with a Master’s degree or higher and a minimum of eight (8) years of experience.

7. PHYSICAL FACILITIES

Every accredited centre shall provide facilities adequate for the functions it is accredited to deliver.

A. Clinical Services (minimum):

Reception and patient waiting area

Assessment and examination room

Fitting and alignment area

Gait training area

B. Fabrication (where applicable):

Casting or digital shape capture area

Socket fabrication area

Assembly and alignment area

Finishing and modification area

C. Support Services (minimum):

Store

Staff area

Patients and staff toilets

Quality control

8. EQUIPMENT & TECHNOLOGY REQUIREMENTS

Facilities must demonstrate the capacity to perform safely: (i) clinical assessment; (ii) fabrication or device modification (where applicable); (iii) dynamic alignment; (iv) rehabilitation and gait training; and (v) quality control and repairs.

Accepted methods and technologies include, but are not limited to:

Plaster-based fabrication systems

Thermoplastic fabrication systems

3D scanning and digital shape capture

3D printing and/or Computer Numerical Control (CNC)-assisted fabrication

Vacuum and lamination systems

Hybrid fabrication approaches

All methods used must meet safety, durability, and clinical outcome requirements.

9. QUALITY AND SAFETY SYSTEMS (MANDATORY)

Every accredited centre shall maintain the following quality and safety systems:

Patient records, prescriptions, and informed consent documentation

Device fabrication logs, component traceability, and delivery records

Personnel Protective Equipment PPE

Infection prevention and control procedures, including cleaning schedules

Structured follow-up and review schedules with documented outcomes

Incident reporting system (adverse events, device failure, skin breakdown)

Complaint handling mechanism and corrective action records

10. PRIVATE AND NON-GOVERNMENT FACILITIES

Private and non-government facilities shall be eligible for accreditation at any level provided they meet the relevant capability, staffing, facility, equipment, and quality requirements. No discrimination shall occur based on ownership.

11. COMPLIANCE AND ENFORCEMENT

Facilities operating outside their accredited scope, or failing to meet quality and safety requirements, shall be subject to downgrading, suspension, or withdrawal of accreditation in accordance with MRTB regulations.

ANNEX A – STANDARD EQUIPMENT & TOOL SCHEDULE

Includes original equipment list and added essential Prosthetic & Orthotic tools, organized by category.

SECTION A – CLINICAL ASSESSMENT & ALIGNMENT TOOLS

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Goniometer | 4 |
| 2 | Protractor | 2 |
| 3 | Measuring tape | 3 |
| 4 | Shoe measuring tape | 3 |
| 5 | Ruler | 2 |
| 6 | Spreading caliper (small & large) | 2 |
| 7 | Vernier caliper (small & large) | 4 |
| 8 | Precision spring compass | 2 |
| 9 | Alignment fixture for prostheses | 4 |
| 10 | Alignment fixture for orthotic joints | 2 |
| 11 | Knee centre jig | 2 |
| 12 | Mounting frame with a laser | 2 |
| 13 | Gait mirror | 1 |
| 14 | Parallel bars | 1 set |
| 15 | Pressure mat | 1 |

SECTION B – CASTING & SHAPE CAPTURE

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Surfoam files (flat, half-round, round) | 3 sets |
| 2 | Plaster knives | 4 |
| 3 | Plaster shears | 2 |
| 4 | Plaster mixing bowls | 4 |
| 5 | Spatulas | 4 |
| 6 | Cast stool | 2 |
| 7 | Sand bath | 1 |
| 8 | Bench vise | 4 |
| 9 | Floor vise | 4 |
| 10 | Mandrel | 2 |
| 11 | 3D scanner | 1 |

SECTION C – FABRICATION

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Allen key set | 4 |
| 2 | T-Handled hex wrench | 4 |
| 3 | Wrench | 2 |
| 4 | Combination spanner set | 2 |
| 5 | Torque wrench | 2 |
| 6 | Torque screwdriver | 2 |
| 7 | Screwdriver set | 4 |
| 8 | Set of pliers | 2 |
| 9 | Wire cutter | 2 |
| 10 | Hammer set | 2 |
| 11 | Hand cabinet file (round, half-round, flat) | 2 |
| 12 | Metal hacksaw frame | 2 |
| 13 | Hacksaw blade | 4 |
| 14 | Sheet metal shears | 2 |
| 15 | Rivet set | 2 |
| 16 | Hole punch set | 2 |
| 17 | Slot punch | 2 |
| 18 | Deburring knife | 2 |
| 19 | Sharpening stone | 2 |

SECTION D – MACHINES

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Floor-mounted drilling machine | 1 |
| 2 | Tabletop drilling machine | 2 |
| 3 | Hand drills (manual & electric) | 2 each |
| 4 | Shoe patching machine | 2 |
| 5 | Shoe stretcher, locking | 2 |
| 6 | Silicon rolling mill | 1 |
| 7 | Dust extraction system | 2 |
| 8 | Compressor | 1 |
| 9 | Vacuum pump | 2 |
| 10 | Hot air oven | 1 |
| 11 | Plastic welding machine | 1 |
| 12 | Jig machine | 1 |
| 13 | 3D printer / CNC | 1 |

SECTION E – LAMINATION

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Workbench for lamination | 2 |
| 2 | Resin mixing station | 1 |
| 3 | Sealing iron | 2 |
| 4 | PVA measuring set | 2 |
| 5 | Vacuum lines & manifolds | 2 |
| 6 | PPE sets | 6 |

SECTION F – QUALITY & SAFETY

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Torque tester | 1 |
| 2 | Load test jig | 1 |
| 3 | Digital weighing scale | 1 |
| 4 | First aid station | 1 |
| 5 | Fire extinguisher | 2 |
| 6 | Emergency eye wash | 1 |

SECTION G – STORAGE & WORKSPACE

| S/N | Equipment | Quantity |
| --- | --- | --- |
| 1 | Tool cabinet/box | 2 |
| 2 | Storage container | 2 |
| 3 | Swivel chair | 2 |
| 4 | Workbench for assembly | 2 |
| 5 | Workbench with drawer unit | 3 |

ANNEX B – Essential Equipment by Accreditation Level

| Functional Area | Level A | Level B | Level C | Level D |
| --- | --- | --- | --- | --- |
| Clinical assessment | Goniometer, measuring tape, vernier caliper, AP/ML gauge, couch, PPE | Same as A | Same as B | Same as C |
| Casting/Shape capture | Plaster tools or a handheld scanner | Full plaster + cast stool | Digital scanner + plaster | High‑res scanner + lab |
| Modification | Surform files, vice | Bench vice, grinder | Router + dust extractor | CNC/advanced router |
| Fabrication | Outsourced allowed | Basic tools | Oven, lamination, vacuum | Multiple ovens, vacuum banks |
| Alignment | Manual jig | Fixtures | Laser fixtures | Digital + laser |
| Repairs | Hand tools | Drill, vice, welding | Lathe + welding | Full machine shop |
| Quality control | Visual + fitting | Measurement tools | Alignment checks | Mechanical testing |

ANNEX C – MRTB CLINICAL ACCREDITATION INSPECTION & SCORING CHECKLIST

This document is used by MRTB inspectors to evaluate Prosthetics & Orthotics facilities for accreditation, renewal, upgrading, or enforcement.

SECTION 1 – LEGAL & ADMINISTRATIVE STATUS (MANDATORY)

1.1 Facility registered with CAC

1.2 MRTB registration for the facility if available

1.3 Valid licenses for all Prosthetists & Orthotists

1.4 Displayed registration certificates

1.5 Waste disposal protocol

1.6 Fire & safety compliance

Score: 0–20

Failure in any item leads to automatic denial.

SECTION 2 – STAFFING & LEADERSHIP

Level A: ≥1 P&O, ≥1 Technician, Head ≥5 yrs

Level B: ≥2 P&O, ≥2 Technician, Head ≥5 yrs

Level C: ≥3 P&O, ≥3 Technician, , Head ≥5 yrs

Level D: ≥5 P&O, ≥5 Technician,  Head MSc + 8 yrs

Score: 0–10

SECTION 3 – FACILITIES

Reception, Assessment room, Fitting area, Gait area, Stores, Staff room, Toilets

Score: 0–10

SECTION 4 – FABRICATION BY TIER

Tier 1: No fabrication

Tier 2: Basic fabrication

Tier 3–4: Advanced fabrication

Score: 0–15

SECTION 5 – EQUIPMENT COMPLIANCE

Verify Annex A & B compliance

Score: 0–20

SECTION 6 – QUALITY & SAFETY SYSTEMS

Patient records, Consent, Fabrication logs, Follow-up, Hazard control, Incident logs, Traceability

Score: 0–15

SECTION 7 – PERFORMANCE/OUTCOME MEASURE

Remake <15%, Skin breakdown <10%, Rejection <10%, Follow-up >80%

Score: 0–10

SECTION 8 – FINAL SCORING

≥80% Full – 4years for public institutions and 3years for private institutions

60–79% Partial - 2years for public institutions and 1years for private institutions

40–59% Restricted - 1year for public institutions and 6months for private institutions

<40% Denied

SECTION 9 – ENFORCEMENT

Upgrade, Downgrade, Suspension, Closure, Disciplinary referral

ANNEX C – Physical Facility Requirements by Level

| Area | Level A | Level B | Level C | Level D |
| --- | --- | --- | --- | --- |
| Reception & waiting | ✔ | ✔ | ✔ | ✔ |
| Assessment room | ✔ | ✔ | ✔ | ✔ |
| Fitting & mirror | ✔ | ✔ | ✔ | ✔ |
| Gait training | Basic rails | Parallel bars | Advanced lab | Instrumented lab |
| Casting area | ✔ | ✔ | ✔ | ✔ |
| Fabrication lab | ✖ | Basic | Full | Multi‑lab |
| Lamination room | ✖ | Optional | ✔ | ✔ |
| Repair section | ✔ | ✔ | ✔ | ✔ |
| Quality control | ✖ | ✔ | ✔ | ✔ |
| Stores | ✔ | ✔ | ✔ | ✔ |
| Toilets | ✔ | ✔ | ✔ | ✔ |
| Seminar Room | ✖ | ✖ | ✔ | ✔ |
| Research lab | ✖ | ✖ | Optional | Mandatory |
| Offices | ✔ | ✔ | ✔ | ✔ |
| Changing room | ✖ | ✔ | ✔ | ✔ |

ANNEX D – ACCREDITATION SCORING, CONDITIONALITIES & DECISION MATRIX

This Annex is issued under the authority of the MRTB and implements the Terms of Reference for Accreditation Guidelines Review Committees, including tiering of practice, realistic requirements, global benchmarks, and performance-based accreditation.

D1. Evaluation Domains

A. Governance & Professional Control

B. Staffing & Competence

C. Clinical & Technical Capability

D. Facilities, Equipment & Safety

E. Quality Systems & Outcomes

D2. Scoring Framework

Each domain is scored out of 20 points. Maximum score = 100.

80–100: Full Accreditation

60–79: Partial Accreditation

40–9: Restricted

Below 40: Denied or Suspended

D3. Mandatory Safety Failures

Automatic denial if:

No licensed Prosthetist/Orthotist in charge

No patient records or consent forms

Unsafe fabrication

Operating beyond scope

D4. Tier-Specific Compliance Thresholds

Level A: 60

Level B: 70

Level C: 80

Level D: 90

D5. Partial Accreditation Rules

Written corrective plan in 30 days, review in 6 months.

D6. Restricted Accreditation Rules

Restricted scope, expires after 6months unless upgraded.

D7. Upgrade & Downgrade Rules

Upgrade after 12 months compliant performance.

D8. Enforcement Actions

Warning, Conditional, Downgrade, Suspension, Withdrawal.

D10. Legal Standing

This Annex implements MRTB ToR and protects public safety.

ANNEX D – Accreditation Scoring & Conditionality Framework

Scoring Domains

| Domain | Percentage |
| --- | --- |
| Staffing & leadership | 10% |
| Legal and administration | 20% |
| Physical facilities | 10% |
| Equipment & technology | 20% |
| Quality & safety | 15% |
| Clinical performance & outcomes | 10% |

Accreditation Decisions

| Score | Outcome | Meaning |
| --- | --- | --- |
| 80–100% | Full Accreditation | Safe, capable, compliant |
| 60–79% | Partial | Minor deficiencies |
| 40–59% | Restricted | Limited scope |
| <40% | Denied | Unsafe for practice |

Signed:

______________________________

Registrar / CEO

Medical Rehabilitation Therapists (Registration) Board of Nigeria (MRTB)